Simulation of Wave Propagation over Reef Flats

Kezhao Fang and Zifeng Jiao

State Key Laboratory of Coastal and Offshore Engineering, Dalian University of Technology, Dalian, P.R. China

Zhongbo Liu

Transportation Management College, Dalian Maritime University, Dalian, P.R. China

Zhili Zou

State Key Laboratory of Coastal and Offshore Engineering, Dalian University of Technology, Dalian, P.R. China

ABSTRACT

This paper describes the simulation of propagation and deformation of regular wave and irregular wave over reef flats, with a shock-capturing numerical model based on second-order fully nonlinear Boussinesq equations. A hybrid high-order finite-volume and finite-difference method is used for wet-dry fronts. In this paper, research on wave height spatial distribution, mean water level changes over reef flats and wave energy spectrum shifts was presented. The numerical results are in close agreement with the physical model experimental data, showing that the numerical model used in this paper has good stability, excellent shock-capturing ability and can tread wet-dry moving boundary. Therefore, this model can well simulate hydrodynamics performance in the process of wave propagation over reef flats.

KEY WORDS:  Boussinesq equations; reef flats; infragravity wave; wave breaking; numerical simulation.

INTRODUCTION

China has a vast maritime territory(Ocean covers most of the planet), and lots of reefs are distributed in waters varying from offshore to open sea. Different from general coastal zone, reef islands usually are just like raised platforms in water, around which there are steep slopes and water depth changes dramatically. The hydrodynamics performance -generated tsunamis generally in a low level compared to earthquake-generated tsunamis, their hazard lies in the destructive waves and the significant coastal runup.

The study of landslides is complex since the movements of underwater masses are natural phenomenon that happens in certain conditions. Generally, the field measurement of tsunami cannot achieve. In order to study the interaction between landslide motion and the propagation and runup of tsunamis, numerous experimental researches and mathematical modeling have been carried on (Grilli and Watts, 1999, 2005; Okal and Synolakis, 2003; Didenkulova et al., 2010; Chubarov et al., 2011). Experimental study into submarine landslide-generated tsunamis began in 1955 with the work of Wiegel (1955). Other experimental researchers have chosen to simulate underwater landslide with a right-triangular block sliding down a 45°slope (Rzadkiewicz et al., 1997; Watts, 2000; Watts and Grilli, 2003). Jiang and LeBlond (1992) appear to be the first to have developed a numerical model that contained the complexities connected with the coupling of a landslide and the waves that it induced. An analytical method to model tsunami generation from rigid submarine landslides was described by Tinti and Bortolucci (2000). The long wave and linear assumptions were invoked. With the same assumptions, Liu et al. (2003) developed an analytical solution for forced long waves on a sloping beach in one horizontal dimension. They compared their analytical results to those obtained from a numerical solution to the non-linear depth-averaged equations. Enet et al. (2003) used a three-dimensional numerical wave tank to model submarine landslides. Sue (2007) designed a two-dimensional configuration which induces a semi-elliptical prism sliding down a planar slope at 15° to the horizontal. And he simulated this landslide experiment using a boundary element method model. It is generally known that the physical experiment method as an important method in studying tsunamis needs professional site and equipment.

The existing mathematical models for modeling landslide-generated waves can be divided into two categories. The first category is the numerical model based on the Navier-Stokes (NS) equations or Reynolds-averaged Navier-Stokes (RANS) equations (e.g., Liu et al., 2005; Yuk et al., 2006) and the second is based on shallow water equations, mainly including the fully nonlinear shallow water (NLSW) equations  model (e.g., Delis and Kazolea, 2011) and Boussinesq equation model (e.g., Lynett and Liu, 2002; Ataie-Ashtiani and Najafi-Jilani, 2006; Dutykh and Kalisch, 2013). The RANS model is very complex and costly in computation. The NLSW model is relatively simple and has higher computational efficiency sine only the related amount of average water depth is calculated. In addition to the advancement in computation, the NLSW model also has many other advantages, such as having a lot of high accuracy numerical computation formats with high resolution. It is also able to handle the wave breaking and the moving shoreline conveniently (Delis and Kazolea, 2011). Considering the above mentioned merits of the NLSW equations, they have been widely used as a simulation tool or basis of theoretical analysis for landslide-generated tsunamis in literature. But this type of model can hardly handle long distance propagation, diffraction effect and other wave effects due to its lack of the frequency dispersion (Ataie-Ashtiani and Najafi-Jilani, 2006; Dutykh and Kalisch, 2013). Boussinesq type models can take frequency dispersion as well as nonlinearity into account while keeping the simplicity of the shallow-water approach. There have been a few attempts to numerical model landslide-generated waves using Boussinesq models (Lynett and Liu, 2002; Ataie-Ashtiani and Najafi-Jilani, 2006; Fuhrman and Madsen, 2009; Zhou and Teng, 2010; Dutykh and Kalisch, 2013). These research show that Boussinesq type models can accurately and efficiently capture the landslide-generated tsunami waves and runup once the numerical tests are within the capability range of the governing equations.

The finite difference (FD) method is the main approach to numerically solving Boussinesq equations due to its simplicity. However several crucial problems with the FD-based solver have been recognized in recent years. First, FD-based Boussinesq model proved to be noisy, or at least weakly unstable to high wavenumbers near the grid Nyquist limit (Shi et al., 2012). Second, too many tuneable variables are introduced in the Boussinesq model, which brings a lot of inconvenience in actual application (Shi et al., 2012; Fang et al., 2013). These problems will become even more severe once the effect of time-dependent bathymetry (emerges as the extra high-order derivatives) is incorporated to consider the landslide-generated waves and runup, especially for higher-order Boussinesq equations. Recent studies show that the hybrid finite volume (FV) and FD method can overcome those defects effectively (e.g., Tonelli and Petti, 2010; Roeber et al., 2010; Orszaghova et al., 2011; Shi et al., 2012; Fang et al., 2013). In such shock-capturing hybrid Boussinesq models, the convective flux terms are discretized using FV method while the rest are approximated using FD method. Wave breaking and wet–dry interface are captured as hydraulic jump and local Riemann problem respectively, which are embodied in the FV-based scheme automatically (Fang et al., 2013). It is thus highly desirable to extend the existing hybrid models to include the effect of time-dependent bathymetry to simulate landslide-generated waves and runup.

In this paper a Boussinesq model for landslide-generated waves and runup is developed, based on the existing shock-capturing Boussinesq model by Fang et al. (2013). The governing equations are firstly extended to include the time-dependent bathymetry. The numerical discretization primarily follows the previous work of Fang et al. (2013). Special attention is paid to reconstruct a well-balanced scheme for both wet cells and dry-wet fronts in order to capture the moving shoreline accurately, while this issue was not well addressed in Fang et al. (2013). Numerous numerical experiments, including both 1DV and 2DH cases, are conducted for model validation and the computed results are compared against the analytical solution or experimental data. The capability of the model in modeling landslide-generated waves and runup are discussed.

GOVERNING BOUSSINESQ EQUATIONS

The two-dimensional extended Boussinesq equations derived by Madsen and Sørensen (1992) read

(1)

(2)

(3)

where the subscripts x, y and t respectively denote the partial derivatives with x, y and time, η is the surface elevation, d(=h +η) is the total water depth, h is the still water depth, P and Q are the velocity flux components in x and y directions, respectively. ψx and ψy are dispersive terms and given by

(4)

(5)

where B is dispersion parameter, the Pade[2,2] approximation of the exact linear dispersion relation leads to B = 1 / 15, which is suitable for medium depth.

The above equations are only valid for wave propagation over fixed seabed. We herein follow the approach in Lynett and Liu (2002) to derive the Boussinesq equations that can take time-varying bathymetry into account. A new still water depth h’(x,y,t) is introduced as

(6)

in which the time-dependent section of the seabed  is assumed to be small. Following the same procedure in Lynett and Liu (2002), we can obtain the Boussinesq equations for submarine landslide

(7)

(8)

(9)

where the dispersion terms ψx and ψy are changed to be

(10)

(11)

Comparing the extended Eqs. (7)-(9) with the original Eqs. (1)-(5), one can find that the effect of the time-dependent emerges in both the continuity equations and momentum equations. Specifically, the first-order derivative of seafloor with respect to time appears in the continuity equation. The second-order derivatives of time-dependent seafloor movement are introduced into the dispersive terms of the momentum equations. It is should be mentioned here that the governing equations primarily keep unchanged except the extra three terms mentioned above under the assumption of Eq.(6), which on one hand brings the simplicity of coding, on the other hand poses the restriction on the application range.

HYBRID SHOCK-CAPTURING SCHEME

In this section the hybrid shock-capturing numerical scheme is presented for completeness and clarity, further details of the numerical discretization are referred to Fang et al. (2013).

Conservative Form of the Governing Equations

The governing equations are rewritten in the conservative forms as

(12)

where U is vector variable, F and G are vector fluxes along x and y direction respectively. They are given by

(13)

with

(14)

(15)

in which, U and V are the combination of the original variable. The new variables water level  and bed elevation zb are introduced in Eqs.(12)-(13) for the convenient implementation of well-balanced scheme proposed by Wang et al. (2011). S in Eq. (12) is the source vector and includes bottom slope (Sb), bottom friction (Sf) and dispersive term (Sd)

(16)

with ψx and ψy rewritten as

(17)

(18)

and

,                     (19)

f in Eq. (19) is the bottom friction coefficient, u and v are the depth-averaged velocities along the x and y direction, respectively.

Well-balanced Hybrid FV and FD Scheme

The conservative governing equations are integrated and discretized on a rectangular cell system. The flux terms and bottom slope terms are solved using Godunov-type FV method while the rest terms are discretized using FD method (Fang et al., 2013). Notably, MUSTA scheme is used for flux evaluation, which is efficient than the usually used HLL scheme or Roe scheme without the loss of accuracy.

However, the well-balanced property at the wet-dry fronts are not well addressed in Fang et al.(2013), where a relatively simple yet rough method (thin film water method) was used and is inadequate for accurately modeling moving shoreline. Here the hydrostatic construction technique proposed by Wang et al. (2011) is used during MUSCL variable construction. The novelty of Wang et al.’s method is the construction of a time-dependent bed elevation and a local modification of bed elevation along wet-dry fronts, which helps to capture the moving shoreline in an accurate and efficient manner. The threshold for defining a dry cell in present study is 10-4 m and, the details of the hydrostatic construction technique are referred to Wang et al. (2011).

Time Integration and Boundary Conditions

The third-order Runge–Kutta scheme with SSP property is applied for time marching. The velocities u and v can then be obtained by solving a tridiagonal system, resulting from discretizing Eqs. (14)-(15) using second-order finite difference formula.

Three ghost cells are set around the entire computation domain. For solid wall, tangential velocity and normal velocity on ghost cells are determined from the inner domain by imposing symmetric and anti-symmetric with respect to the solid we-dry interface. Once the local ratio of wave height to water depth exceeds 0.8, waves are identified as breaking and the Boussinesq equations are switched to NSWE at the corresponding cells.

For all the numerical tests, the time-dependent bottom is prescribed and does not change its shape during sliding, as in previous Boussinesq models (Lynett and Liu, 2002; Ataie-Ashtiani and Najafi-Jilani, 2006; Fuhrman and Madsen, 2009; Zhou and Teng, 2010).

MODEL VALIDATIONS AND DISCUSSIONS

Several numerical simulations, including both 1DV and 2DH cases, are performed in this section for model validation. The capabilities and limitations of the model are also discussed.

One Dimensional Waves Generated by a Solid Slide

The first test problem is taken from Delis and Kazolea (2011), which is a 1D case of waves generated by a solid submarine slide moving with a constant speed over a horizontal frictionless bed. The initial landslide profile is sinusoidal with the maximum height 20 m and length 1500 m. The initial total water depth is 100 m, the slide velocity is 5 m/s and the total computational domain is 20000 m. The analytical solution of this problem is available in Tinti et al. (2001), which is a superposition of three waves. At the first stage of wave generation, a positive and negative wave are produced due to the abrupt initiation of the slide at t=0 and travel with celerity of c=±(gd)1/2 away from the source of generation. Then a third smaller trough wave appears and propagates with the same celerity as the landslide.

In the simulation, we used gird size △x=4.0 m, time step △t=0.001 s and bottom friction was neglected. Fig.1 shows the comparison between the numerical and analytical results. One can see that the simulated results are in general agreement with the analytical solution, denoting the development and evolution of the three waves are reasonably well reproduced by the model. The arrival time for the three waves predicted by the model compares well with the analytical solution, with only a slight lag develops after 60 seconds. The modelled amplitudes are also in good agreement with the analytical solution with an apparent overestimation for the third smaller trough wave. The numerical results presented in Fig.1 also demonstrate the well-balanced scheme is successfully implemented, according to Delis and Kazolea (2011).

The tail waves following the first two waves are also observed from the numerical results in Fig.1, they whilst do not appear in the analytical solution. The discrepancy is caused by the inconsistency between the governing equations for analytical solution and numerical model, namely, the form uses linear nondispersive shallow water equations while the latter uses the nonlinear and dispersive Boussinesq equations.

Fig.1 Comparison of analytical results and numerical results for 1D wave generation and propagation by a sinusoidal slide.

Propagation and runup of the Tsunami Induced by 1D Landslide

The next test case is taken from Liu et al. (2003), which is for tsunami wave propagation due to a one-dimensional landslide over a slope. This test has been used as a benchmark test in the Third International Workshop on Long-Wave Runup Models and all related data are also provided, see Liu et al. (2008) for details. Our aim here is to test the ability of the numerical model to predict the free surface elevation and runup.

The water depth in this test is composed of two components as

(20)

where=5.7° is the beach slope, x is the horizontal axis starting from the initial shoreline position and positive offshore and h’ is the time-dependent function of the seabed, given by

(21)

where d is the maximum vertical slide thickness, m is the ratio of the thickness and the slide length, and the landslide moves at constant acceleration.

Two cases will be simulated using the developed Boussinesq model and, the simulation parameters are listed bellow (f=0)

- Case A: tanm =10 with m =0.01 (very thin mass layer), d =1. The computational domain is 5000 m in length and discretized using gird size △x=2.0 m. The time step is set to be △t=0.001 s.

- Case B: tanm =1.0 with m =0.1(relatively thick mass layer)), d =1. The computational domain is 130 m in length and discretized using gird size △x=0.1 m. The time step is set to be △t=0.003 s.

Liu et al. (2003) obtained the analytical solution to this problem based on the linear shallow water equations, which will be compared against the computed results at the nondimensional instances:  =0.5, 1.0, 1.5 for the first case and 0.5, 1.0, 2.5, 4.5 for the second one.

Fig.2 Tsunami generation and runup: Initial setup and the surface elevation for case A at times =0.5, 1.0, 1.5.

Fig.3 Tsunami generation and runup: surface elevation for case B at times =0.5, 1.0, 1.5, 4.5.

Fig.2 shows the comparison between computed results with analytical solutions for case A. The computed wave profiles are in good agreement with analytical solutions. The time-varying runup is also produced well except a slight difference close to the shoreline area. Fig.3 shows the comparison between computed results with analytical solutions for case B. As shown in this figure, the computed results and the analytical solutions are close at the early time since comparing to the linear driving force the accumulating influence of nonlinearity are small (Delis and Kazolea, 2011). But for later times, the nonlinear propagation effects become important and then the difference between numerical and analytical results increases, the similar trend has been observed in other landslide models (Delis and Kazolea, 2011, Liu et al. 2008). The nonlinear terms, which are omitted in deriving the analytical solution, become stronger in time and the analytical solution is not adequate for this case (Delis and Kazolea, 2011). The numerical results for this test confirm the importance of nonlinear terms for the thick slide with relatively short length scale.

Modelling the Tsunami Wave Generated by a Three-dimensional Smooth Slide

In this section, a three-dimensional submarine landslide tsunami laboratory experiments reported by Enet and Grilli (2007) will be simulated. Fig. 4 is the sketch of the experimental setup of the experiments, which were performed in a 3.7 m wide, 1.8 m deep and 30 m long wave tank. The key geometrical parameters are given in Fig. 5. The Gaussian shape landslide model has length b, width w, thickness T and is initially located at x=xi with a submergence depth d.

Fig.4 General view of experimental setup of Enet and Grilli’s (2007) for submarine landslide (taken from Enet and Grilli, 2007).

Fig.5 Vertical cross section for tsunami landslide experiments (taken from Enet and Grilli, 2007).

According to Enet and Grilli (2007), the thickness T=0.082 m, the length b=0.395m and the width w=0.680 m. The submarine mass slides down a q=15°slope and is stopped by a foam cushion placed at the bottom of the slope, where the maximum water depth is 1.5 m. This Gaussian shape landslide can be described by

(22)

with kb=2C/b, kw=2C/w, C=acosh(1/e), and the truncation parameter e=0.717. A time-dependent formula for water depth was derived by Fuhrman and Madsen (2009) and used in the present simulation

(23)

where is computed by next iterative equation for a given x, y and t:

(24)

with

, ,                  (25)

in which x0 is the initial x position of the submarine sliding mass, S(t) is the resulting motion parallel to the slope, a0 is the initial landslide acceleration and ut is the terminal velocity. Further details of the Eqs.(23)-(25) are referred to Fuhrman and Madsen (2009).

Three cases of Enet and Grilli’s experiments are simulated to test and verify the Boussinesq-type model. The initially minimum submergence depth at the rest sliding mass is different for three cases, i.e., d=61 mm, 120 mm, 189 mm respectively. The key parameters of landslides for each case are listed in Table 1, where xg is the initial position of the minimum submergence xg=x0+Tsinq.

Table 1. Parameters for three cases of the three-dimensional underwater landslide simulations, as given by Enet and Grilli (2007).

| Case | d | xg | a0 | ut |
| --- | --- | --- | --- | --- |
| 1 | 61 mm | 551 mm | 1.20 m/s2 | 1.70 m/s |
| 2 | 120 mm | 763 mm | 1.17 m/s2 | 2.03 m/s |
| 3 | 189 mm | 1017 mm | 1.21 m/s2 | 1.97 m/s |

Since the y-axial symmetry of the bottom, the submarine slide mass and the landslide-generated waves, only the top half area is simulated. The computational dimension for all these three cases is 7.5 m in length and 1.875 m in width and discretized using gird size △x=0.025 m. Time step △t=0.001s and bottom friction is also neglected.

Four snapshots of the computed free surfaces for the case with d=61 mm at instants t=0.5, 1.5, 2.5 and 3.5 s are presented in Fig.6 to give an intuitive understanding of the generation and evolution of waves generated by the submerged slide mass. It should be noted here that the amplitudes of waves in Fig.6 are magnified 5 times for illustration aim while the seabed is in the original size. It can be seen from the figure that the generated-tsunami waves contain a relatively long leading wave and a series of short waves behind the first wave. This wave pattern is inconsistent with the experimental picture taken by Enet and Grilli (2007) and numerical results from Fuhrman and Madsen (2009), providing an initial qualitative (visual) validation.

In the experiments of Enet and Grilli (2007), free surface elevations were collected at four gauge positions. For all cases, the location of the first gauge is (x, y) = (x0, 0) m with the x0 is the initial position of the submarine slide mass given in Table 1. The rest three gauge are kept at fixed positions, i.e., (x, y) = (1.469, 0.350) m for gauge 2, (x, y) = (2.289, 0) m for gauge 3 and (x, y) = (1.929, 0.4988) m for gauge 4. Figs.7~9 show the comparison between computed results with the experimental data of the free surface at four gauge locations. For all three cases, the Boussinesq model has almost identical performances. The computed results at gauge 1 agree well with the measurements (where dispersion is negligible as landslide-generated waves propagate offshore), both the maximum depression and the arrival time are reasonably produced. For all the gauges, the free surface elevations are also well captured by the model at the initial stage. After that however, significant deviation from the experimental data are observed both in amplitude and phase. The discrepancy is mainly attributed to the weak dispersion of the model. According to Lynett and Liu (2002), the ratio of the side length of the landslide (=b/2 approximately) to the water depth must be greater than 3.5 to ensure the numerical tests are within the application range of the depth-averaged Boussinesq model, while they are approximately 3.24 for Case 1, 1.65 for Case 2 and 1.04 for Case 3 at the initial position of the slide mass and even smaller at the gauge locations. Regarding of this, it is not surprising to see the apparent mismatch in Figs. 7-9. Boussinesq models with much accurate dispersion indeed overcome these problems and present satisfactory numerical results (e.g., Ataie-Ashtiani and Najafi-Jilani, 2006; Fuhrman and Madsen, 2009; Zhou and Teng, 2010).

Fig.6 Computed free surface for the submarine landslide case with d=61 mm at time 0.5 s, 1.5 s, 2.5 s and 3.5 s.

Fig.7 Computed and experimental data of free surface elevations for Case 1 (d=61 mm).

Fig. 8 Computed and experimental data of free surface elevations for Case 2 (d=120 mm). For gauge 3, only numerical results are plotted due to the unavailability of the measurements.

Fig. 9 Computed and experimental data of free surface elevations for Case 3 (d=189 mm).

CONCLUSIONS

An existing shock-capturing Boussinesq model is extended to include the effect of time-dependent bathymetry to simulate the waves and runup induced by sliding masses. Landslide is assumed as a rigid body without changing its shape during sliding and in the simulation the trajectory of the slide mass is prescribed. Numerous numerical experiments, including both 1DV and 2DH cases, are performed. For 1DV cases, the computed results from the model generally agree with the analytical solutions derived from linearized shallow water equations, denoting its capability in predicting landslide generated waves and runup. The nonlinearity is found to play a key role in the vicinity of the shallow water region, as shown by the 3rd 1DV test. The 2DH test presented here is quite challenging as it requires high accurate dispersion. The present Boussinesq model can only reproduce the initial state of wave generation while fails in predicting the dispersion-dominated stage. We hence conclude that the accurate nonlinearity and dispersion is highly desirable for simulating landslide-generated waves and runup.

The proposed model inherits the advantages of a shock-capturing hybrid scheme. Within the application range, it is expected to be an operational tool for predicting landslide generated waves and runup due to its simplicity and superiority to NLSW equations in dispersion and nonlinearity. Further numerical investigation using the develop model is undergoing.

ACKNOWLEDGEMENT

The authors would like to acknowledge financial support by the National Foundation for Creative Research Groups (Grant No: 51221961).

REFERENCES

Ataie-Ashtiani, B, and Najafi-Jilani, A (2006). "Prediction of Submerged Landslide Generated Waves in Dam Reservoirs: An Applied Approach," Dam Eng, 17(3), 135.

Chubarov, LB, Khakimzyanov, GS, and Shokina, N (2011). “Numerical Modelling of Surface Water Waves Arising Due to Movement of Underwater Landslide on Irregular Bottom Slope,” Computational Science and High Performance Computing Iv, Springer, 75-91.

Delis, A, and Kazolea, M (2011). "Finite Volume Simulation of Waves Formed by Sliding Masses," Int J Num Methods in Biomedical Eng, 27(5), 732-757.

Didenkulova, I, Nikolkina, I, Pelinovsky, E, and Zahibo, N (2010). "Tsunami Waves Generated by Submarine Landslides of Variable Volume: Analytical Solutions for a Basin of Variable Depth," Natural Hazards and Earth System Science, 10(11), 2407-2419.

Dutykh, D, and Kalisch, H (2013). "Boussinesq Modeling of Surface Waves Due to Underwater Landslides," Nonlin. Processes Geophys., 20, 267-285.

Enet, F, and Grilli, ST (2007). "Experimental Study of Tsunami Generation by Three-Dimensional Rigid Underwater Landslides," J Waterway, Port, Coastal, and Ocean Eng, 133(6), 442-454.

Enet, F, Grilli ST, and Watts, P (2003).  "Laboratory Experiments for Tsunamis Generated by Underwater Landslides: Comparison with Numerical Modeling," Proc 13th Int Offshore and Polar Eng Conf, ISOPE03, Hawaii , 3, 372-379.

Fang, K, Zou, Z, Dong, P, Liu, Z, Gui, Q, and Yin, J (2013). "An Efficient Shock Capturing Algorithm to the Extended Boussinesq Wave Equations," Appl Ocean Res, 43, 11-20.

Fuhrman, DR, and Madsen, PA (2009). "Tsunami Generation, Propagation, and Runup with a High-Order Boussinesq Model," Coast Eng, 56(7), 747-758.

Grilli, ST, and Watts, P (1999). "Modeling of Waves Generated by a Moving Submerged Body. Applications to Underwater Landslides," Eng Anal Bound Elem, 23(8), 645-656.

Grilli, ST, and Watts, P (2005). "Tsunami Generation by Submarine Mass Failure. I: Modeling, Experimental Validation, and Sensitivity Analyses," J Waterway, Port, Coastal, and Ocean Eng, 131(6), 283-297.

Jiang, L, and LeBlond, PH (1992). "The Coupling of a Submarine Slide and the Surface Waves Which It Generates," J Geophysical Research: Oceans (1978–2012), 97(C8), 12731-12744.

Liu, P-F, Wu, T-R, Raichlen, F, Synolakis, C, and Borrero, J (2005). "Runup and Rundown Generated by Three-Dimensional Sliding Masses," J Fluid Mech, 536, 107-144.

Liu, PL-F, Lynett, P, and Synolakis, CE (2003). "Analytical Solutions for Forced Long Waves on a Sloping Beach," J Fluid Mech, 478, 101-109.

Liu, PL, Yeh, HH, and Synolakis, C (2008). Advanced Numerical Models for Simulating Tsunami Waves and Runup, World Scientific.

Lynett, P, and Liu, PL-F (2002). "A Numerical Study of Submarine–Landslide–Generated Waves and Run–Up," Proc Royal Society of London. Series A: Mathematical, Physical and Eng Sciences, 458(2028), 2885-2910.

Madsen, PA, and Sørensen, OR (1992). "A New Form of the Boussinesq Equations with Improved Linear Dispersion Characteristics. Part 2. A Slowly-Varying Bathymetry," Coast Eng, 18(3), 183-204.

Okal, EA, and Synolakis, CE (2003). "A Theoretical Comparison of Tsunamis from Dislocations and Landslides," Pure Appl Geophys, 160(10-11), 2177-2188.

Roeber, V, Cheung, KF, and Kobayashi, MH (2010). "Shock-Capturing Boussinesq-Type Model for Nearshore Wave Processes," Coast Eng, 57(4), 407-423.

Rzadkiewicz, SA, Mariotti, C, and Heinrich, P (1997). "Numerical Simulation of Submarine Landslides and Their Hydraulic Effects," J Waterway, Port, Coastal, and Ocean Eng, 123(4), 149-157.

Shi, F, Kirby, JT, Harris, JC, Geiman, JD, and Grilli, ST (2012). "A High-Order Adaptive Time-Stepping Tvd Solver for Boussinesq Modeling of Breaking Waves and Coastal Inundation," Ocean Modelling, 43, 36-51.

Sue, LP (2007). "Modelling of Tsunami Generated by Submarine Landslides."

Tinti, S, and Bortolucci, E (2000). "Analytical Investigation on Tsunamis Generated by Submarine Slides."

Tinti, S, Bortolucci, E, and Chiavettieri, C (2001). "Tsunami Excitation by Submarine Slides in Shallow-Water Approximation," Pure Appl Geophys, 158(4), 759-797.

Tonelli, M, and Petti, M (2010). "Finite Volume Scheme for the Solution of 2d Extended Boussinesq Equations in the Surf Zone," Ocean Eng, 37(7), 567-582.

Watts, P (2000). "Tsunami Features of Solid Block Underwater Landslides," J Waterway, Port, Coastal, and Ocean Eng, 126(3), 144-152.

Watts, P and Grilli, ST (2003).  "Underwater Landslide Shape, Motion, Deformation, and Tsunami Generation," Proc 13th Offshore and Polar Eng Conf, ISOPE03, Hawaii, 3, 364-371.

Wiegel, RL (1955). "Laboratory Studies of Gravity Waves Generated by the Movement of a Submerged Body," Transactions, American Geophysical Union, 36, 759-774.

Yuk, D, Yim, S, and Liu, P-F (2006). "Numerical Modeling of Submarine Mass-Movement Generated Waves Using Rans Model," Comput Geosci-uk, 32(7), 927-935.

Zhou, HQ, and Teng MH. (2010). " Extended Fourth-Order Depth-Integrated Model for Water Waves and Currents Generated by Submarine Landslides," Journal of Eengineering Mechanics, 136(4), 506-516.